Department of Electrical and Computer Engineering

COEN 6501                                                                               Dec. 8, 2014

Answer all Questions.                                All Questions carry equal marks

Exam Duration 3 hour

No books, papers are allowed.                         Lecturer: Asim J. Al-Khalili

===============================================================

- Question  1

- Implement F using 2:1 MUXs only. All inputs should be of non-inverting type.

F = ABC’ + AB’C’ + ABC+ A’B’C

- Implement the same functions using look up tables.

Note: You are allowed to minimize F.

- Question 2

Design a hardware using carry save adder arrays to calculate y.
y= N2 + N +1
N is an unsigned binary number 4 >N>0.  Evaluate your design in terms of speed and area

Question 3

Design a shift and add multiplier controller as shown in Fig.1. The controller has two inputs:

L:  the least significant bit of the multiplier operand, “0” or”1”

N:  Counter output “0” or”1”

The controller has one output.        S:  Stop signal “0” or”1”

Operation of the controller is as follows:

The controller is usually in the idle state.

If L is ‘1’, then an add and shift operation is performed, If LSB is ‘0’ then a shift operation is performed.  In both cases, the counter counts up by one step.

When the counter reaches it desired value N=1, a Stop signal S=1 is generated and the state machine goes to the stop state. Design the sequential circuit starting with a state diagram

Note: The counter and the reset mechanism are designed separately.

Shift and Add

L

N

S

Fig.1

Question 4

a. Identify all the paths in the circuit shown in the Fig. 2 below.

b. Determine maximum speed of operation at typical conditions for the

Timing parameters for all components are listed in Table 1.

c. At the maximum speed of operation, determine the slack time for the setup time and hold time at the D-input of Flip-Flop U2.

D

D     Q

Q

D     Q

Z

CLK

U2

U5

U6

U7

U8

U9

U10

U11

U3

U1

U4

Fig. 2

| - Component | - Tp (ns) | - Input Loading (UL) | - K1 ns/UL | - K2 ns/fanout |
| --- | --- | --- | --- | --- |
| - Inverter | - 0.15 | - 1 | - 0.1 | - 0.08 |
| - NAND/NOR(2input) | - 0.24 | - 2 | - 0.05 | - 0.14 |
| - NAND (3 input) | - 0.4 | - 1.5 | - 0.12 | - 0.18 |
| - Flip Flop, ↑, (CK to Q) Tsu=1 ns, th = 0.5ns | - 1.5 | - 2 | - 0.1 | - 0.2 |

Table 1

Question 5

a. The Circuit shown in Fig. 2 operates at a frequency 50 MHz. Signal A arrives at  -∞. Determine the arrival time, the required time and the slack time at points B and C.

b. Determine the maximum clock frequency, considering worst case scenario.

The circuit is implemented on a die, packaged in a ceramic DIP with a thermal resistance of 30oC/W.  Power consumption of the chip is 1.5 W with room temp at 25C. Fanout loading is neglected. Use Table 2 parameters.

c. Determine the maximum frequency of operation if skew is −2ns.

D1    Q

D2    Q

Q

Q

D1    Q

D1    Q

CLK

D2    Q

CLKd

D2    Q

CLK

D2

D3    Q

0.2ns

0.4ns

0.3ns

U3

U1

U2

A

B

C

| - Component | - Tp (ns) | - Input Loading (UL) | - K1 ns/UL |
| --- | --- | --- | --- |
| - AND/(2input) | - 0.2 | - 1 | - 0.15 |
| - OR (2 input) | - 0.3 | - 2 | - 0.25 |
| - XOR (3 input) | - 0.4 | - 3 | - 0.35 |
| - Flip Flop, ↑, (CLK to Q) Tsu=1 ns, th = 0.5ns , tskew=2ns | - 2 | - 4 | - 0.45 |

Fig. 2

,       K’ = KT * KV *K                                           KT =  ,         TJ = Tamb + Φ Ja  * PdKV =     ,          KP = 1+ 0.01 * fP

Table 2

Question 6

For the circuit shown below,

- Write a structural VHDL code for the entity “Adder”.

- Write behavioral VHDL code, “Test_Bench” for testing the circuit

Test_Bench

Adder

Generator

- Draw a response to the following VHDL Code

process

begin

B<= not A ;

wait until A=‘1’ for 10 ns;

end process;

A

10	 20	  30	   40	    50	     60

B -------------------------------------------------------------?

=====================================================================

Appendix A

1. tcs,max   < tCQmin + tCLmin + tsu.min

2. Tmin   tCQmax + tCLmax+ tsUmaxR2 –tcsmin

3. thmaxR2 < tCQmin + tCLmin - tcsmax

tCL = tLogic + tinterconnec